Transforming between ITRF and NZGD2000

9 May 2017

linz.govt.nz

Contents

1	Introduction	3

2	Transformation from other ITRF realisations to ITRF96	4

3	Conversion between ellipsoidal and Cartesian coordinates	6

4	Using deformation model to transform ITRF96 to NZGD2000	7

5	Example transformation	8

6	References	8

- Introduction

New Zealand Geodetic Datum 2000 (NZGD2000) is aligned with the International Terrestrial Reference Frame 1996 (ITRF96) and referenced to epoch 2000.0.  However the NZGD2000 coordinates also include the deformation due to earthquakes since 2000, which have been applied as "reverse patches" in the NZGD2000 deformation model.

The procedure for converting coordinates from an ITRF realisation, for example ITRF2008,  at a given date, to NZGD2000 is to first convert the ITRF2008 coordinate to ITRF96 and then subtract the deformation calculated from the NZGD2000 deformation model at that date.

Conversely, to calculate an ITRF coordinate at a specific date from an NZGD2000 coordinate at a specific date the deformation derived from the NZGD2000 deformation model is added to the coordinate to obtain an ITRF96 coordinate, which is then converted to the required ITRF realisation.

- Transformation from other ITRF realisations to ITRF96

The relationship between various ITRF realisations is defined by the International Earth Rotation and Reference Systems Service (IERS) and is available at http://itrf.ensg.ign.fr/doc_ITRF/Transfo-ITRF2014_ITRFs.txt. This document defines the relationships between each ITRF realisation and ITRF2014 at epoch 2010.0 (note that this epoch differs from that used for the table below, which is epoch 2000.0).

The IERS did not determine any significant difference between ITRF96 and ITRF97. However the International GNSS Service (IGS) derived a non-zero transformation between ITRF96 and ITRF97 (Soler and Snay, 2004).  This differs because the IGS used only GPS (rather than including other space techniques) and with GPS only data the difference was statistically significant.  Since the NZGD2000 datum is primarily based on GPS observations, the IGS derived relationship is adopted for New Zealand.

The transformation parameters from recent ITRF realisations to ITRF96 are listed below.  The values are defined as the value at 2000.0, and the change per year.  To convert from ITRF96 to another realisation the values are negated.  (Note that Soler and Snay use an opposite sign convention for the rotations Rx, Ry, and Rz to that used by IERS – the table below follows the IERS convention).

| ITRF | Tx (mm) | Ty (mm) | Tz (mm) | S (ppb) | Rx (mas) | Ry (mas) | Rz (mas) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Tx (mm/yr) | Ty (mm/yr) | Tz (mm/yr) | S (ppb/yr) | Rx(mas/yr) | Ry(mas/yr) | Rz (mas/yr) |
| ITRF2014 rates | 6.4 0.79 | 3.99 -0.6 | -14.27 -1.44 | 1.08901 -0.07201 | -0.16508 -0.01347 | 0.26897 0.01514 | 0.11984 0.01973 |
| ITRF2008 | 4.8 | 2.09 | -17.67 | 1.40901 | -0.16508 | 0.26897 | 0.11984 |
| rates | 0.79 | -0.6 | -1.34 | -0.10201 | -0.01347 | 0.01514 | 0.01973 |
| ITRF2005 | 6.8 | 2.99 | -12.97 | 0.46901 | -0.16508 | 0.26897 | 0.11984 |
| rates | 0.49 | -0.6 | -1.34 | -0.10201 | -0.01347 | 0.01514 | 0.01973 |
| ITRF2000 | 6.7 | 3.79 | -7.17 | 0.06901 | -0.16508 | 0.26897 | 0.11984 |
| rates | 0.69 | -0.7 | 0.46 | -0.18201 | -0.01347 | 0.01514 | 0.01973 |
| ITRF97 | 0 | -0.51 | 15.53 | -1.51099 | -0.16508 | 0.26897 | 0.05984 |
| rates | 0.69 | -0.1 | 1.86 | -0.19201 | -0.01347 | 0.01514 | -0.00027 |
| ITRF96 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| rates | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

The coefficients are scaled by the following factors in the transformation equations below:

cT = 0.001 (millimetres to metres, applies to Tx, Ty, Tz)

cS= 1.010-9 (ppb to ratio, applies to S)

cR = /(18060601000)=4.8481410-9 (mas to radians, applies to Rx, Ry,Rz)

To transform from an ITRF realisation, for example ITRF2008, to ITRF96, the transformation parameters are first determined at the epoch of the transformation.  The parameter Tx at time t (in years) is determined as:

|  | δt=t-2000 Tx,t=Tx+δt.Tx | Eqn. 1 |
| --- | --- | --- |

The other parameters are similarly calculated to the transformation epoch.

The transformation is then applied by converting the ITRF coordinate longitude, latitude, and ellipsoidal height to geocentric Cartesian coordinates, then applying the transformation, and then converting the resulting Cartesian XYZ coordinates back to longitude, latitude, and ellipsoidal height.  The conversions between Cartesian and ellipsoidal coordinates are detailed below.

The formula for transforming from Cartesian coordinates (Xn, Yn, Zn) in realisation n  of ITRF to ITRF96 Cartesian coordinates (X96, Y96, Z96) is:

|  | X96Y96Z96 =XnYnZn +cT.Tx,tcT.Ty,tcT.Tz,t +cS.St-cR.Rz,tcR.Ry,tcR.Rz,tcS.St-cR.Rx,t-cR.Ry,tcR.Rx,tcS.St.XnYnZn | Eqn. 2 |
| --- | --- | --- |

To transform from another ITRF96 to another ITRF realisation the signs of all the parameters are reversed, so the formula is:

|  | XnYnZn =X96Y96Z96 +-cT.Tx,t-cT.Ty,t-cT.Tz,t +-cS.StcR.Rz,t-cR.Ry,t-cR.Rz,t-cS.StcR.Rx,tcR.Ry,t-cR.Rx,t-cS.St.X96Y96Z96 | Eqn. 3 |
| --- | --- | --- |

- Conversion between ellipsoidal and Cartesian coordinates

The conversions between longitude, latitude, and ellipsoidal height λ, θ,h and geocentric Cartesian coordinates X,Y,Z use the parameters of the GRS80 ellipsoid:

Semi-major axis a = 6378137.0 metres

Reciprocal of flattening 1/f = 298.257222101

To convert ellipsoidal to geocentric coordinates use the following formulae:

|  | b=1-f.a | Eqn. 4 |
| --- | --- | --- |
|  | ρ=a2cosθb2sin2θ+a2cos2θ+hcosθ | Eqn. 5 |

|  | X=ρcosλ Y=ρcosλ Z=b2sinθb2sin2θ+a2cos2θ+hsinθ | Eqn. 6 |
| --- | --- | --- |

There are no simple formulae to convert Cartesian coordinates to ellipsoidal coordinates – this requires an iterative solution to determine the latitude and height.  The steps are as follows:

Calculate the longitude and a first approximation to the latitude θ0

|  | λ=tan-1YX ρ=X2+Y2 θ0=tan-1a2Zb2ρ | Eqn. 7 |
| --- | --- | --- |

Then the following equation is used to derive successively better approximations to the latitude θ1, θ2, … until successive iterations converge to the desired tolerance

|  | θi+1=tan-1Z+a2-b2sinθib2sin2θi+a2cos2θi/ρ Repeat until θi+1-θi< 10-10 | Eqn. 8 |
| --- | --- | --- |

The height is determined from the final latitude θ as

|  | h=ρcosθ+Zsinθ-b2sin2θ +a2cos2θ | Eqn. 9 |
| --- | --- | --- |

- Using deformation model to transform ITRF96 to NZGD2000

To convert ITRF96 coordinates at a specific epoch to NZGD2000 coordinates the deformation component at the observation epoch must be subtracted from the coordinates.  The NZGD2000 deformation model is used to calculate this component based on the location (longitude/latitude) of the point and the epoch.

The NZGD2000 deformation model calculates the deformation in terms of east, north, and up displacements (δet, δnt, δut) in metres at time t relative to NZGD2000 reference coordinates at the time of observation.  The east and north offsets must be converted to degrees δλt, δθt in order to apply them to the latitude and longitude. (Note that the east/north offsets are defined on the surface of ellipsoid, not at the elevation of the observation point).

|  | δλt=δetb2sin2θi+a2cos2θia2cosθ 180π | Eqn. 10 |
| --- | --- | --- |
|  | δθt=δntb2sin2θi+a2cos2θi32a2b2 180π | Eqn. 11 |

To convert the ITRF96 reference coordinates to NZGD2000 coordinates at time t, these components are subtracted from the coordinates, ie

|  | λNZGD2000=λ96-δλt θNZGD2000=θ96 -δθt hNZGD2000=h96-δut | Eqn. 12 |
| --- | --- | --- |

- Example transformation

By way of an example we will transform an ITRF2008 coordinate

LLHITRF2008 = (174.774752, -41.284944, 48.52)

observed on 27 April 2013 to NZGD2000.

Applying Eqn. 6 the longitude/latitude/ellipsoidal height transforms to a geocentric Cartesian coordinate

XYZITRF2008 = (-4779860.9786, 437125.2533, -4186286.2229)

The date 27 April 2013 is expressed equivalent to year 2013.32.  The transformation parameter Tx at this epoch is therefore

Tx,2013.32 = 4.8+(2013.32-2000.0)0.79 = 15.3228mm.

The other parameters Ty,t,  Tz,t,  St,  Rx,t,  Ry,t and Rz,t are calculated for the epoch in a similar way as 5.902mm, -35.5188mm, 0.0502368ppb, -0.3445004mas, 0.4706348mas, and 0.3826436mas  respectively.

Using these parameters in Eqn. 2 the coordinate converts to an ITRF96 coordinate

XYZITRF96 = (-4779860.9739, 437125.2316,  -4186286.2485)

and applying Eqn. 7 to Eqn. 9 this has latitude, longitude, and height

LLHITRF96 = (174.774752252, -41.284944213, 48.5319)

The NZGD2000 deformation model must be evaluated at this location and time to transform this to NZGD2000.  Using version 20130801 of the deformation model this evaluates to:

(δe,δn,δu) = (-0.2717, 0.4346, 0.0000)

Applying the scaling from Eqn. 10 and Eqn. 11 this converts to

δλ,δθ,δh = (-0.00000324, 0.00000391, 0.0)

And subtracting this deformation from the ITRF96 coordinates gives the final NZGD2000 coordinates as

LLHITRF96 = (174.77475550, -41.28494813, 48.5319)

- References

Soler, T. & R.A. Snay (2004). Transforming positions and velocities between the International Terrestrial Reference Frame of 2000 and North American Datum of 1983, J. Surv. Engrg., ASCE, 130(2), 49-55. Available at http://geodesy.noaa.gov/CORS/Articles/SolerSnayASCE.pdf